ESP32-CAM Security Camera with Motion Detection & Telegram Alerts

This guide will help you build a low-cost smart security camera using ESP32-CAM and a PIR motion sensor, which sends alerts (with photos) via Telegram and also provides live video streaming over Wi-Fi.

- 🧰 Components Required

- ESP32-CAM Module (OV2640 camera)

- PIR Motion Sensor (HC-SR501)

- ESP32-CAM USB Programmer

- 2 × 18650 Li-ion Batteries + Battery Holder

- DC-DC Buck Converter (LM2596) – adjustable or fixed 5V

- Breadboard or Custom PCB

- JST Connectors (optional)

- Arduino UNO (for PIR sensor calibration – optional)

- Screws, spacers, double-sided tape

- Multimeter

- 🧪 What This Project Does

- Detects motion using PIR sensor

- Captures a photo on detection

- Sends the photo instantly to your Telegram

- Streams live video from ESP32-CAM over Wi-Fi

- 🛠️ Step-by-Step Build Guide

- ✅ 1. Upload Code to ESP32-CAM

- Connect ESP32-CAM to USB programmer via header pins.

- Use Micro-USB to connect it to your PC.

- Install Arduino IDE from Microsoft Store.

- Add ESP32 Board Support:

  - Go to File → Preferences

  - In "Additional Board Manager URLs", paste:
https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Go to Tools → Board Manager, search ESP32, click Install

- Select ESP32 Wrover Module as the board.

- Open the example: File → Examples → ESP32 → Camera → CameraWebServer

- Paste your modified code (with Telegram functionality) into this sketch.

- ✅ 2. Telegram Setup

- In Telegram, search BotFather, type /newbot

- Create a bot → copy the token.

- Search for chat ID bot → start it → copy your Chat ID.

- Paste your Wi-Fi, Telegram token, and chat ID into the code.

- ✅ 3. Final Code Prep

- Choose a GPIO pin for PIR sensor (example: GPIO 3)

- Set Tools:

  - Partition Scheme: Huge App

  - Board: ESP32 Wrover Module

  - COM Port: (your ESP32’s port)

Click Upload.

- 📡 4. Live Stream & IP Address

- After uploading, open Serial Monitor, baud rate: 115200

- Press reset button on ESP32-CAM

- Copy the IP address shown — this is your live stream URL.

- 🔌 5. Circuit Connections

- PIR Sensor → ESP32-CAM GPIO (e.g., GPIO 3)

- ESP32-CAM 5V & GND → from Buck Converter Output

- Buck Converter Input → from 7.4V (2 × 18650 batteries)

- Use multimeter to set exact 5V output on buck converter

- Use double-sided tape, PCB, spacers to neatly mount components

⚠️ Don’t power ESP32-CAM directly from 7.4V — it will burn! Always step it down to 5V.

- 🧪 6. PIR Sensor Calibration (Optional with Arduino)

- Upload basic motion detection sketch to Arduino UNO

- Connect PIR sensor’s signal pin to Arduino pin 2

- Calibrate:

  - Left knob: Sensitivity (distance)

  - Right knob: Time delay (turn counterclockwise for short time)

- Test with onboard LED → once working, use it in final ESP32 circuit.

- 📲 7. Testing the Setup

- Power the device using batteries or 5V supply

- Move in front of PIR sensor

- You should receive a photo alert in Telegram

- Visit IP address → view live stream

- 🛠️ Troubleshooting Tips

Problem: Streaming crashes or lags
Fix:

- Set XCLK to 8 MHz in stream settings

- Lower camera resolution